|  | Petroleum and petrochemistry Time:2 hr 319 Chem. 28/12/2015 | Benha University Faculty of Science Chemistry Department |
| --- | --- | --- |

الإجابة النموذجية لامتحان كيمياء البترول و البتروكيماويات

319  ك

(نصف ورقة امتحانية)

الفرقة : الثالثة

الشعبة : جميع الشعب المزدوجة و الجيولوجيا و الجيوفيزياء

التاريخ : السبت  26 / 12 /  2015

الممتحن : د/ محمد عبد الرحمن موسى ابو ريا

قسم : الكيمياء

كلية : العلوم

Chose the correct answer:

- ………… processes are used to improve the qualities and quantities of the product.

- Treating.               b- Conversion.      c- Separation.             d- Distillation.

- The crude oils is being formed by the action of steam or water on metallic carbides in the inner portion of earth crust known as:

- Organic theory.      b- Modern theory.        c- Inorganic theory.     d- Molecular theory.

- ………… , an aromatic hydrocarbon, is one of the petroleum compositions.

- Thiophene.                     b- Propane.                   c- Indan.                      d- Cyclohexane.

- Separating a mixture of gases by absorbing the gases with a liquid solvent is known as:

a- Absorption method.                       b- Distillation method.                                                                               c- Solvent extraction method.              d- Gases method.

- The percent of sulfur in most crude oils is ranging between………..

- 1 : 8 %.                           b- 0.5: 3 %.                   c- 0.5 : 10 %.               d- 3 : 7 %.

- Crude oils which contain paraffin wax but little or no asphaltic matter is called……

- Mixed-base crude oils.                             b- Paraffin-base crude oils.                                                                      c- Asphaltic-base crude oils.                        d-Wax-base crude oils.

- …………. is the lowest temperature at which the oil is completely miscible with equal volume of aniline.

a- Kraft point.                    b- Flame point.          c- Aniline point.          d- oil temperature.

- The presence of some optically active compounds in the higher boiling fractions of many crude oils is evidence that the origin of crude oils is:

- Inorganic.                    b- Metallic.                  c- Organic.                  d- Chemically.

- Destroying the emulsion of water with crude oils by using demulsifiers is known as:

- Thermal method.      b- Chemical method.     c- elec tric method.        d- water method.

- Separation processes are based on the differences in the ………… properties.

a- chemical.                b- physical.                       c- electrical.                   d- mechanical.

- ………… is one of the atmospheric distillation products.

- Gasoline.              b- Lubricating oils.           c- Bitumen.                      d- Asphaltic.

- …….……… is using as solvent to remove aromatic components from kerosene.

- Sulphuric acid.             b- Cyclohexane.          c- Pentane.                      d- Water.

- Octane number is the percentage of iso-octanein a mixture of iso-octane and …….

a- iso-heptane.                   b- n-heptane.                 c-benzene.                    d-cyclohexane.

- ……..is the percentage of cetane in a mixture of cetane with 1-methylnaphthaline.

- Octane number.      b- Naphthaline number.   c- Cetane number.       d- Diesel number.

- The cleavage of hydrocarbon molecules done by the action of high temperature known as……….

- Catalytic cracking.   b- Treating process.       c- Reforming.       d- Thermal cracking.

- The purpose of ………… is to improve the antiknock characteristics of gasoline.

- cocking.                   b- pyrolysis.                     c- reforming.       d- thermal cracking.

- …….. processes is used to remove the undesirable impurities of the finished petroleum products.

- Treating.                   b- Hydrofining.              c- Separation.              d- Refining.

- High molecular paraffinic hydrocarbons react with hydrogen to form two or more hydrocarbons known as………

- hydrocarcking.   b- isomerization.        c- dehydrogenation.        d- dehudrocyclization.

- The polymerization of butylenes is used for the production of……………

- polymeric gasoline.   b- Polymeric octane.    c-polymeric surfactants.     d- polymeric paraffin.

- ……….. processes is used to remove high concentration of hydrogen sulphide by organic amines.

- hell.                       b- Girbotol.                  c- Sweetening.                         d- Phosphate.

- Converting of mercaptans in gasoline to disulphides is known as………. process.

- dehydrosulphurization.     b- Girbotol.           c- sweetening.                d- mercaptans.

- Dehydrodenitrogention used to convert nitrogen compounds into…….

- secondary amines.            b- tertiary amines.    c-pyridine.                   d-ammonia.

- Treating the petroleum products with hydrogen in the presence of catalyst is known as………

- hydrofining.             b- sweetening.            c-conversion.              d-catalytic reforming.

- The Discovery of methane and some paraffinic gases in atmosphere of different  planets, is evidence that the origin of crude oils is:

- Inorganic.                            b- Metallic.                c-Organic.                   d-Chemically.