|  | Petroleum additives Time:2 hr 413 Chem. 26/12/2015 | Benha University Faculty of Science Chemistry Department |
| --- | --- | --- |

الإجابة النموذجية لامتحان كيمياء الإضافات البترولية

413 ك

(نصف ورقة امتحانية)

الفرقة : الرابعة

الشعبة : الكيمياء الخاص و الكيمياء  و الجيولوجيا

التاريخ : السبت  26 / 12 /  2015

الممتحن : د/ محمد عبد الرحمن موسى ابو ريا

قسم : الكيمياء

كلية : العلوم

Chose the correct answer:

- Base oil has ……………….

- freezing point at 0OC b) freezing point at 10OC.  c) no freezing point.d) freezing point at -10OC.

- Paraffinic base oil  have ………. fluidity at low temperature.

- poor.                            b) high.                                c) good.                   d) moderate.

- An increase in temperature………..the viscosity of lubricating oil and vice versa.

- increase.                       b) reduce.                           c) improve.               d) have no effect on.

- The rate of change in viscosity with temperature can be expressed in terms of……………..

- Thermal viscosity.      b) kinematic viscosity.        c) viscosity index.   d) viscosity.

- The lowest temperature at which it can vaporize to form an ignitable mixture in air is ………….

- kraft point.                  b) ignite point.                    c) Aniline point.       d) flash point.

- The ……….is the temperature at which wax crystals begin to appear in oil liquid.

- kraft point.                  b) ignite point.                   c) Aniline point.         d) cloud point.

- In crude oil a high pour point is generally associated with high………….content.

- paraffin.                      b) aromatic.                        c) naphthenic.             d) alcoholic .

- The lowest temperature at which a mineral oil is completely miscible with an equal volume of aniline is………….

- kraft point.                 b) ignite point.                     c) Aniline point.         d) flash point.

- Methods commonly used to measure the aromatic content of oil are………...

- IR-method and ASTM D 2140.   b) density.         c) viscosity.                d) a and b.

- ………….is the ratio of the dynamic viscosity to the density of the fluid.

- Thermal viscosity.     b) kinematic viscosity.         c) viscosity index.     d) viscosity.

- The oxidation reaction rate of oil is doubles when the temperature is raised by ……………°C.

- 10.                             b) 20.                                    c) 30.                          d) 50.

- One of the following compounds isn't a product of the lubricating oil   oxidation.

- Resins.                   b) Acidic compounds.         c) Lacquers.                 d) Alkali compounds.

- The oxidation reaction mechanism of oil is……………. mechanism.

- free radical.             b) anionic.                           c) cationic.                    d) carbocation.

- …………is the tendency to form carbon residue under high temperature conditions in an inert atmosphere.

- Carbon residue.       b) Corrosion.                       c) Decomposition.       d) Oxidation.

- ……………..additives are designed to clean the metal surfaces within a fired engine and prevent the build-up of deposits.

- Dispersants.           b) Corrosion.                         c) Anti-wear.               d) Detergents.

- A dispersant molecule consists of …………

- hydrocarbon group.     b) polar group.                c) connecting group.    d) a,b and c.

- Higher in molecular weight of dispersants than the organic portion (soap) of the detergent made it …………. effective in fulfilling the suspending and cleaning functions than detergents.

- less.                     b) more.                              c) equal.                         d) a,b and c.

- Detergents and dispersant can …………

- reduce wear.        b) decrease engine life.       c) reduce oxidation.       d) a,c.

- The dispersants …………. deposit precursors in oil.

a) suspend.               b) coagulate.                       c) aggregate.                   d) precipitate.

- Oxidation generally ……………oil viscosity.

- increase.              b) reduce.                            c) not effect on.              d) decrease.

- The radical scavengers are known as ……….. antioxidants.

- primary.              b) secondary.                       c) tertiary.                        d) a and b.

- The peroxide decomposers are known as ………...

- Peroxide termination.   b) oxygenated.          c) secondary antioxidants. d) free radical.

- …………..they are gasoline-soluble chemicals mixed with gasoline to enhance certain performance.

a) Gasoline additives.     b) Oil additives.             c) Oxidation additives.        d) Free radical.

- …………. can degrade antiknock quality, cause fuel pump wear, and attack plastic or elastomeric fuel system parts.

a) Antioxidant.                b) Oil additives.             c) Lacquers.                         d) Peroxides.

- ……….. prevent free water in gasoline from rusting or corroding fuel system parts.

a) Corrosion inhibitors.   b) Oil additives.             c) Lacquers.                         d) Antioxidant.

- ……………they are chelating agents that capture specific metal ions.

a) Corrosion inhibitors.   b) Metal deactivators.    c) Lacquers.                          d) Antioxidant.

- ……………improve the water-separating characteristics of gasoline by preventing the formation of stable emulsions.

a) Detergents.                  b) Metal deactivators.    c) Lacquers.                          d) Demulsifiers.

- Carboxylic acids and carboxylates can be used as………...

a) Detergents.                  b) Metal deactivators.    c) Corrosion inhibitors.        d) Demulsifiers.

- ……………increase the antiknock quality of gasoline.

a) Phenolic compounds.  b) Oxides.                      c) Lead alkyls.                       d) Zinc oxides

- Ferrocene compounds  are used as……………

a) Corrosion inhibitors.  b) Demulsifiers.              c) Antiknocking.                    d) Antioxidant.

- ……………..number is represents as the percentage by volume of isooctane (2,2,4-trimethylpentane) in a fuel mixture of isooctane and heptane.

a) Cetane.                       b) Isooctane.                   c) Heptane.                              d) Octane.

- The most common oxygenated compounds used as anti-knock additives is ………

a) Lead alkyls.                b) methyl tertiary-butyl ether.  c) isopropanol.              d) acetone.

- Silver corrosion inhibitors are substituted of ………...

a) thiadiazole.                 b) imidazole.                 c) furan.                                    d) qunoline.

- Maximum allowable concentration of aromatics in gasoline as anti-knock additives is…………. volume.

a) 25%.                            b) 35%.                        c) 45%.                                       d) 20%.

- The average concentration of lead in gasoline reached a maximum of about ……..milligrams per liter.

a) 660.                             b) 1000.                      c) 450.                                          d) 770.

- Octane number is a measure of the …………….properties of the fuel.

a) gasoline.                       b) antioxidant.           c) antiknock.                                d) wear.

- ………….they prevent gasoline components from reacting with oxygen in the air to form peroxides or gums.

a) Corrosion inhibitors.    b) Demulsifiers.       c) Oxidation inhibitors.             d) Metal deactivators.

- Copper/ silver corrosion inhibitors additives act as film formers, which prevent ………. from reaching and react with the surface.

a) sulfur.                           b) oxygen.                 c) zinc.                                      d) lead.

- A method for measuring the loss in mass after 22 hours evaporation at a certain  temperature (often 107°C) is ………..

a) pour point.                    b) fire point.             c) oil volatility.                         d) flash point.

- The solubility of base oil can be indicated by……………..

a) Viscosity Gravity Constant.         b) Aniline point.       c) carbon residue.     d) a and b.